Governance operators and viability kernels of the Edwards Aquifer: an intervention-selection test at J-17

Applied management analysis

Prepared for submission to Journal of Water Resources Planning and Management

Author information withheld for double-anonymous peer review.

Corresponding-author details to be completed by the author at submission.

30 August 2026

Contribution statement. Scored robust-kernel analysis of pumping-rule families on the J-17 record shows reactive critical-period rules deliver 3.3–16.2% more permitted supply at matched robust protection, with the cascade matching the deepest cut's protection at 50.6% more supply.

Keywords. drought triggers; critical-period management; robust viability; pumping policy; Edwards Aquifer

# Abstract

This study follows a preregistered retention rule: intervention selection must be scored, not asserted — a governance module is kept only if it improves a preregistered protection-and-supply outcome. This paper runs that test on the San Antonio Pool of the Edwards Aquifer at the J-17 index well: robust viability kernels of declared safe sets for a declared pumping-policy family, under persistent drought-floor recharge classes, with an erosion conversion — the device that converts a declared model defect into a certified-kernel erosion margin from the closed loop’s contraction rate — applied in the contraction form the fitted one-pool affine map admits. The affine map is fitted by ordinary least squares on 1934–1990 and audited out of sample on 1991–2023; the governance family comprises business-as-usual pumping, flat caps, a Stage-I reactive rule, and a critical-period management (CPM) cascade.

Results. (1) Physical threshold (618 ft, Comal cessation proximity): under a perpetual drought-of-record floor, business-as-usual is not robustly viable (its nominal kernel empties beyond T≈13 years; the T=12 boundary is 692.6 ft), every flat cut of 10% or deeper — and the two reactive rules — makes the whole declared safe set robustly invariant, and the smallest flat cut securing the threshold is 7.2% of mean pumping. (2) Institutional threshold (660 ft, post-2007 Stage I): a negative certificate — a machine-verified finding of non-retention or certified non-existence, distinct from a statistical null result — every declared policy’s robust kernel equals business-as-usual’s at every horizon, because the viable region lies strictly above the first trigger level; the institutional threshold is protected by wet years, not by the declared pumping family. (3) Certified layer: with the training-defect erosion applied, every demand-management policy’s certified kernel is empty beyond T=3 years (zero pumping retains one through T=4 under the drought-of-record floor and T=5 under the milder floors) — the binding constraint on certified intervention claims is the model defect, not the governance. (4) Positive selection: the reactive rules are retained — the Stage-I rule matches the flat caps’ robust invariance at 3.3–16.2% more permitted supply, and the cascade matches the deepest cut’s protection at 50.6% more. A 1950s open-loop replay records the map’s drought bias (8.1 ft, high) without correction. The paper’s deliverable is the scored comparison on one measured system; no two-pool, karst, or solute claim is made.

# 1. Introduction

Drought management in permitted groundwater systems is built on triggers: index-well or springflow thresholds at which pumping reductions activate. Trigger design has a long evaluation literature — drought indicators and triggers are properly assessed as stochastic objects, with explicit attention to how indicator levels, trigger thresholds, and drought stages interact (Steinemann 2003) — and robustness has been a recurring concern of water-resources planning since the recognition that optimal solutions can be brittle to the hydrologic assumptions that produce them (Watkins and McKinney 1997). The operational question for a district that already operates triggers is narrower and comparative: given a declared family of pumping rules — business-as-usual, flat caps, and reactive trigger rules — which rules deliver protection under adverse recharge, and at what cost in permitted supply?

This study answers that question by scoring rather than assertion. It follows a preregistered retention rule under which a governance module is kept only if it improves a declared protection-and-supply outcome. Robust viability kernels — the sets of initial states from which the closed-loop system keeps the state inside a declared safe set under every disturbance in a declared class — are computed for each declared pumping rule under persistent drought-floor recharge classes; permitted supply is replayed on the observed head record; and a certified layer converts the fitted map’s declared defect into an erosion margin. The complete comparison — object, defect declaration, disturbance classes, policy family, and retention rule — was frozen in a dated protocol (2026-08-26) before any score was computed.

We selected the Edwards (Balcones Fault Zone) Aquifer J-17 index well because its long (1934–2023) daily head record, its explicit institutional thresholds, and its management relevance make it a natural test bed for scored policy evaluation. The San Antonio Pool is a karst aquifer whose regional behavior has long been represented with lumped and equivalent-porous-media models (Scanlon et al. 2003); the scored object here is the simplest such representation, a one-pool affine head map fitted on the historical record.

The governance question is the intervention leg of a paired evaluation. A companion forecast-evaluation study (under separate review) scored prediction modules on the same J-17 series and ended in a negative certificate — persistence beats the causal ladder, and the oracle water-balance gap (7.55 ft against persistence’s 13.23 ft; 7.55 ft against the retained AR(1)’s 12.84 ft) locates the unexplained forecast gap in the information layer. This study asks whether a declared governance operator changes the viability kernel of the real system, and at what cost in permitted supply.

The remainder of this article is organized as follows. Section 2 states the object, the fitted dynamics, the information pattern, the safe sets, and the governance family. Section 3 states the erosion conversion. Section 4 reports the results: worst-case attractors and the minimal cut (Section 4.1), nominal kernels (Section 4.2), certified kernels (Section 4.3), supply and retention (Section 4.4), and classification and stress replays (Section 4.5). Section 5 interprets the findings, Section 6 states the limitations, and data and code availability and the references close the paper.

# 2. Object, dynamics, information

The object is that of the companion forecast evaluation: J-17 annual-mean head zt (ft AMSL), San Antonio Pool, 1934–2023, from the fixed twenty-column analysis panel (data/annual_panel.csv). z is a measured well level, not an assessment inversion. Recharge R (USGS/EAA estimated) and pumpage P (EAA Table 1, San Antonio Pool wells) are the fluxes. No new data.

Dynamics: the causal stock-flow class of the companion forecast study (its M2), one pool, affine —

ΔHt=α+βRt+γPt+δHt−1,

fitted by OLS on 1934–1990 (56 transitions). OLS fit: α=163.49, β=0.0198 ft per 103 acre-ft, γ=−0.02844 ft per 103 acre-ft, δ=−0.2539, a=1+δ=0.7461 — a contraction with a 25.4%-per-year mean reversion. Residual SD 5.60 ft, max 15.41 ft (training). Out-of-sample (1991–2023, audit only): SD 8.40, max 21.81 ft.

Information pattern: the manager ends year t knowing Ht,Rt,Pt and sets Pt+1=πHt; Rt+1 is unknown at decision time and is treated adversarially within a declared persistent floor (UC-min = 43.7, UC-q05 = 166.5, UC-q10 = 179.1 103 acre-ft yr​−1 — the drought-floor recharge classes; the 1956 drought-of-record year is UC-min). These floors are certification geometry — the persistent floor regimes are harsher than any single recorded year (UC-min is the recorded 1956 minimum, held perpetually) — not recharge forecasts.

Safe sets (both declared normative thresholds, fixed in the frozen protocol): Kphys*=618 ft (Comal cessation proximity) and Kinst*=660 ft (post-2007 Stage I trigger; not applied to pre-2007 history).

Governance family: BAU (P≡P=282.16, training mean); flat caps ρP, ρ∈{0.9,0.8,0.7,0.6,0.5,0}; the Stage-I reactive rule (20% cut when H<660, the reduction verified against the Authority’s published rule); and a critical-period management (CPM) cascade (cumulative 20/30/35/40% cuts at H<660/650/640/630 — Stage I verified, stages II–IV declared normative-threshold scenarios).

# 3. Erosion conversion (discrete-contraction form)

With uniform defect ε=15.41 ft (the training maximum) and contraction a=0.7461, trajectory deviation over T years is bounded by rT=ε1−aT/1−a: r1=15.41, r3=35.49, r5=46.66, r∞=60.70 ft. The certified kernel at horizon T is the nominal kernel of K*+rT. The out-of-sample defect max (21.81 ft) exceeds the declared ε: the certified rows below are optimistic out-of-window; this is recorded, and no refitting is performed, per protocol.

# 4. Results

## 4.1 Worst-case attractors and the minimal cut

The worst-case (UC floor) attractor of the closed loop, by policy:

| Policy | UC-min | UC-q05 | UC-q10 |
| --- | --- | --- | --- |
| BAU | 615.72 | 625.31 | 626.29 |
| flat-90% | 618.88 | 628.47 | 629.45 |
| flat-80% | 622.04 | 631.63 | 632.61 |
| S1 (reactive 20% < 660) | 622.04 | 631.63 | 632.61 |
| flat-70% | 625.20 | 634.79 | 635.77 |
| CPM cascade | 628.36 | 636.37 | 637.35 |
| flat-60% | 628.36 | 637.95 | 638.93 |
| flat-50% | 631.52 | 641.11 | 642.09 |
| flat-0 (zero pumping) | 647.32 | 656.91 | 657.90 |

Under a perpetual 1956-recharge floor, BAU’s attractor (615.72 ft) sits below the physical threshold; the smallest flat cut whose attractor clears 618 ft is 7.2%. The Stage-I reactive rule’s attractor equals flat-80%’s (the cut is active on the entire attractor branch).

## 4.2 Nominal kernels (no erosion)

Under the physical threshold (618 ft) and UC-min, BAU’s kernel boundary climbs 618.8 (T=1) → 625.6 (T=5) → 658.4 (T=10) and the kernel is empty beyond T≈13 years — the T=12 boundary is 692.6 ft and the T=13 kernel is empty (the continuous crossover is 12.7 under the same convention used for the zero-pumping horizons below) — BAU is not robustly viable against a perpetual drought-of-record. Every cut policy in the family (a 10% flat cap, S1, CPM, and deeper) makes 618,710 robustly invariant: the whole declared safe set is the kernel at every horizon, including the infinite horizon. Under UC-q05/q10 the safe set is already invariant at BAU (attractors 625.3 / 626.3 ft): governance differentiates the kernel only under the drought-floor class.

Under the institutional threshold (660 ft): negative certificate. Every declared policy’s robust kernel equals BAU’s at every horizon — the boundaries (675.1 at T=1, 695.3 at T=2 under UC-min; empty from T=3) lie strictly above the first-stage CPM trigger (660 ft — the highest of the four declared trigger levels, and therefore the easiest to reach), so no declared demand-management rule activates in the viable region and the kernel is policy-invariant. Demand management extends the viable horizon, not invariance: even zero pumping has an empty nominal kernel beyond T≈6 (UC-min) / T≈11 (UC-q10), because its worst-case attractor (647.3 / 657.9 ft) still sits below 660 ft. The institutional threshold is protected by wet years, not by the declared pumping family — which is exactly the frequency-management rationale the actual CPM rule implements, and that rationale is outside the robust-kernel frame.

## 4.3 Certified kernels (eroded)

With the erosion of Section 3 applied, every demand-management policy in the family has an empty certified kernel beyond T=3 years at the physical threshold and beyond T=1 year at the institutional threshold. Zero pumping is the exception: its certified physical-threshold kernel is nonempty through T=4 under UC-min (lower boundary ≈687.9 ft from the affine certified-kernel algebra; the committed horizon grid {1,2,3,5,8,10,15,20,∞} does not sample T=4, so the archived T=∞ emptiness is consistent with the analytic boundary exceeding the domain top at T=5) and through T=5 under UC-q05/q10 (physical threshold only). The certified boundaries at T=3 / UC-min / 618 ft: flat-0 662.2 < flat-80 697.8 < BAU = S1 = CPM 706.7. The binding constraint on certified intervention claims is the model defect, not the governance — the information-layer limitation identified by the companion forecast evaluation, here measured on the intervention leg.

## 4.4 Supply and retention

Mean prescribed pumping (actual-head replay, 1934–1990):

| Policy | Supply (103 acre-ft yr​−1) | Cut active |
| --- | --- | --- |
| BAU | 282.16 | 0% |
| flat-90% | 253.94 | 100% |
| flat-80% | 225.73 | 100% |
| flat-70% | 197.51 | 100% |
| S1 | 262.36 | 35.1% |
| CPM | 254.93 | 35.1% |
| flat-60% | 169.29 | 100% |
| flat-50% | 141.08 | 100% |

Out-of-sample replay (1991–2023, audit only): S1 264.5 and CPM 260.6 103 acre-ft yr​−1; every flat policy prescribes its cap throughout, so the training and out-of-sample supplies coincide.

Retention rule (frozen): at least as protective as BAU everywhere, and more water than the most protective flat cap with matched protection.

- S1: RETAINED (nominal, under UC-min at the 618 ft threshold). It matches the flat caps’ robust invariance (kernel = whole safe set, all horizons) while supplying 262.4 vs flat-90%’s 253.9 (+8.4 103 acre-ft yr​−1, +3.3%) and flat-80%’s 225.7 (+36.6, +16.2%). The reactive architecture justifies its additional structure: the same protection at strictly more permitted supply.

- CPM: RETAINED (nominal, same threshold and class). Attractor 628.4 (equal to flat-60%’s) at supply 254.9 vs flat-60%’s 169.3 (+50.6%).

- Certified level: S1 and CPM remain retained against their dominating flat caps (S1 vs flat-80: +36.6 supply at every certified horizon; CPM vs flat-60: +85.6), but only over the T≤3 horizons where their certified kernels are nonempty.

- Under the institutional threshold: nothing retained (all policies ≡ BAU).

## 4.5 Classification and stress replays

T=5 nominal kernel (UC-min, 618 ft): BAU excludes exactly one actual year from its viable set — 1956, the drought-of-record year (623.15 ft annual mean). S1 and CPM exclude none: the entire 90-year actual record is robustly 5-year viable under the cut rules. The T=5 certified kernels are empty, so no actual year is certified 5-year viable under any policy; this is the boundary of the certified analysis.

1950s open-loop diagnostic (model with actual R,P vs actual heads, 1951–1956): the affine map under-predicts the drought decline — model 659.5 → 631.3 vs actual 659.5 → 623.2, max error 8.1 ft, biased high. The 1950s model-based policy replays (from the observed 1950 head) keep all policies above 618 ft (BAU min 629.7, S1 634.9, CPM 637.1), but the open-loop bias means the true margins are smaller than the replay suggests; this is recorded, and no correction is applied.

# 5. Interpretation

The complete evaluation loop — measured state, calibrated stock-flow map, declared governance operators, declared uncertainty classes, viability kernels with explicit declared-defect erosion, held-out defect audit, and a fixed retention rule — yields a positive selection result: the reactive architecture matches flat-cap protection at 3–50% more permitted supply, and a 7.2% mean cut secures the physical threshold against a perpetual drought-of-record where BAU fails. It also yields two negative findings: the institutional threshold is not demand-manageable to invariance under the declared classes, and the certified content is defect-bound to T≤3 years. The unexplained forecast gap lies again in the information layer: the erosion bound absorbs the oracle gap measured against the retained AR(1) baseline (12.84 versus 7.55 ft).

Two layers of results must be kept distinct. The robust-kernel findings are statements about the declared pumping family under the declared floor classes: business-as-usual is not robustly viable beyond T≈13 years under the perpetual-1956 floor (the T=12 boundary is 692.6 ft), every cut policy makes the whole declared safe set robustly invariant at the physical threshold, and the institutional-threshold negative certificate holds for every declared policy. The certified-kernel findings are a different layer, produced by applying the erosion conversion to the declared training defect: with ε=15.41 ft (exceeded out of sample at 21.81 ft), every demand-management policy’s certified kernel is empty beyond T=3 years. Neither layer’s claims paraphrase the other; in particular, the certified emptiness is a bound on what the fitted map can certify, not a robustness statement about the physical system.

# 6. Limitations

Nominal kernels carry no defect margin; certified kernels use a training defect that the out-of-sample audit exceeds (15.4 vs 21.8 ft). The map is one-pool affine on annual means; the actual CPM triggers are 10-day averages, so the annual-mean rule is a coarse relative of the real institution. Stage II–IV reductions are declared scenarios, not verified. San Antonio + Uvalde are lumped (inherited defect). Observation error is not separated from model defect. The UC floors are certification geometry, not forecasts; the 1950s replay is biased high by 8.1 ft. Nothing in this leg promotes or demotes any forecast module, and no two-pool, karst, or solute claim is made.

# Data and code availability

All input data, analysis scripts, and result files are archived in the research programme’s public repository at https://github.com/MIKEAA2020/general-sustainability; an anonymized copy is available for double-anonymous review. J-17 daily highs: Texas Water Development Board well 6837203. Recharge: Umphres and Choi (2025), DOI 10.5066/P1BI62NY. Pumpage: Edwards Aquifer Authority (2024/25), Table 1. The full intervention protocol — object, defect declaration, disturbance classes, policy family, and retention rule — was frozen and dated (2026-08-26) before any kernel, boundary, replay, or retention score was computed; it is archived with the analysis code as the preregistration record, alongside the companion forecast-evaluation protocols dated 2026-08-25.

python3 src/run_intervention.py

The analysis is fully deterministic (no random components). Re-executing the committed runner regenerates both output files, results/intervention_results.json and results/intervention_boundaries.csv; a verification re-execution in a fresh environment reproduced both files byte for byte. The machine-readable outputs include the fields retention (nominal), retention_certified, and certified_horizon_nonempty in the JSON. A corrected runner (src/run_intervention_v2.py) fixes the retention comparator’s treatment of empty kernels (an empty kernel now counts as worst on both sides, matching the companion study’s convention; the earlier asymmetric comparator was mechanically verified inert for these artifacts) and writes results/intervention_results_v2.json and results/intervention_boundaries_v2.csv, value-identical to the committed outputs; the committed original files are retained unchanged as the audited record.

# References

Edwards Aquifer Authority. 2024/25. 2023 Groundwater Discharge and Usage. Table 1, after USGS letter report 5 April 2024. https://www.edwardsaquifer.org/wp-content/uploads/2025/05/2023-Groundwater-Discharge-and-Usage.pdf

Edwards Aquifer Authority. Critical Period / Drought Management. https://www.edwardsaquifer.org/groundwater-users/critical-period-drought-management/

Puente, C. 1978. Method of estimating natural recharge to the Edwards Aquifer in the San Antonio area, Texas. U.S. Geological Survey.

Scanlon, B. R., Mace, R. E., Barrett, M. E., and Smith, B. 2003. Can we simulate regional groundwater flow in a karst system using equivalent porous media models? Case study, Barton Springs Edwards aquifer, USA. Journal of Hydrology 276: 137–158.

Steinemann, A. C. 2003. Drought indicators and triggers: a stochastic approach to evaluation. Journal of the American Water Resources Association 39: 1217–1233.

Texas Water Development Board. Water Data for Texas, well 6837203 (J-17). https://waterdatafortexas.org/groundwater/well/6837203

Umphres, G. D., and Choi, N. J. 2025. Estimated Annual Recharge to the Edwards Aquifer in the San Antonio Area, by Stream Basin or Ungaged Area, 1934–2024. U.S. Geological Survey data release. https://doi.org/10.5066/P1BI62NY

Watkins, D. W., Jr., and McKinney, D. C. 1997. Finding robust solutions to water resources problems. Journal of Water Resources Planning and Management 123: 49–58.